# INSTRUCTION MANUAL

AI-Based Laser Eye Gaze Detection and Assistive Control System

# 1. System Overview

This system is designed to enable hands-free human–computer interaction using eye gaze tracking. It uses computer vision techniques to detect eye movement and convert it into physical control signals. The system consists of a webcam-based vision module, a processing unit, and an ESP32-controlled pan–tilt laser mechanism. It is intended for assistive applications, especially for individuals with limited mobility.

# 2. Safety Precautions

• Do not point the laser directly into the eyes.
• Ensure a stable 5V power supply to avoid malfunction.
• Avoid loose wiring to prevent short circuits.
• Operate the system in a safe indoor environment.

# 3. Installation Guide

Step A: Vision Setup
• Connect the webcam and place it at eye level (40–60 cm).
• Ensure proper lighting for accurate detection.
• Run the gaze detection software.

Step B: Hardware Setup
• Power the ESP32 using a regulated supply.
• Connect pan and tilt servos.
• Mount the laser pointer on the servo platform.
• Establish serial communication between PC and ESP32.

# 4. System Operation

• The camera captures real-time facial images.
• MediaPipe detects facial and iris landmarks.
• Gaze direction is classified into left, right, up, down, or center.
• Commands are transmitted to ESP32.
• Servo motors adjust position and move the laser accordingly.

# 5. Blink Detection

Blink detection is implemented using the Eye Aspect Ratio (EAR). When the EAR value drops below a threshold, a blink is detected. Intentional blinks are used as trigger commands for system interaction.

# 6. Output

• Laser pointer indicates the gaze direction.
• Servo motors provide smooth pan–tilt motion.
• System operates in real time with minimal delay.

# 7. Troubleshooting

• No detection: Check camera connection and lighting.
• Lagging: Reduce resolution or close background apps.
• Servo not responding: Check wiring and power supply.
• Incorrect output: Recalibrate camera position.

# 8. Maintenance

• Clean camera lens regularly.
• Maintain consistent lighting conditions.
• Inspect servo alignment periodically.
• Ensure all connections remain secure.

TRANSMITTER SIDE                                                                                         RECEIVER SIDE